# Lab 4 Singularity Observation Report

EEL 4664 Robotics

Observe three straight stylus motions: a well-conditioned baseline, an approach to a straight-elbow singularity, and an approach to a shoulder singularity. Compare where the robot moves rapidly even though Cartesian speed remains constant.

Name: ______________________________________________

Section or team: _____________________________________

Date: _______________________________________________

# Prediction

Before running Webots, predict which robot geometry and joint motion will identify the elbow and shoulder singularities in Trials 2 and 3.

________________________________________________________________________________

________________________________________________________________________________

# Recorded Results

Copy the three values printed by the singularity_demo controller for each trial.

| Quantity | Trial 1 Move away undamped | Trial 2 Approach elbow singularity | Trial 3 Approach shoulder singularity |
| --- | --- | --- | --- |
| Minimum singular value (sigma_min) |  |  |  |
| Condition number |  |  |  |
| Peak commanded joint rate (rad/s) |  |  |  |

Interpret the recorded quantities as follows:

Minimum singular value (sigma_min): larger is better. Zero means the Jacobian is singular; near zero means one Cartesian motion direction is almost lost.

Condition number: smaller is better. One is ideal; a large value means the inverse is sensitive and can strongly amplify joint rates.

Peak commanded joint rate: larger means faster joint motion. Compare it with the 3.00 rad/s Webots simulation limit.

# Webots Evidence

Insert one screenshot for each trial. Capture Trial 2 near its straight-elbow endpoint with the final singularity metrics and joint-rate readouts visible. Capture Trial 3 during its continuous shoulder-singularity crossing with the on-scene geometry and joint-rate readouts visible. The controller automatically generates the joint-rate comparison plot while the trials run. After Trial 3, read the complete saved location printed in the Webots console. In Word, choose Insert > Pictures > This Device and select lab04_singularities/joint_rate_comparison.svg, then place it in the final placeholder below.

## Trial 1 Move Away Undamped Baseline

Paste a screenshot of Trial 1 here.

## Trial 2 Approach Elbow Singularity Undamped

Paste a screenshot of Trial 2 near the straight-elbow endpoint here. Keep the singularity metrics and joint-rate readouts visible.

## Trial 3 Approach Shoulder Singularity Undamped

Paste a screenshot captured during the continuous Trial 3 crossing here.

## Joint Rate Comparison Plot

Paste the generated joint_rate_comparison.svg here. The plot should show all three trials and the 3.00 rad/s simulation limit.

Paste the generated joint-rate comparison plot here. File: lab04_singularities/joint_rate_comparison.svg

# Questions

Answer each question in two or three sentences and refer to your recorded results.

## 1  What happened to the minimum singular value and condition number as Trial 2 approached the straight elbow

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

## 2  Why did the undamped joint rate rise near the end of Trial 2 even though Cartesian speed remained constant

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

## 3  What visual evidence shows that Trial 3 crosses a shoulder singularity rather than another elbow singularity

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

## 4  Why was it necessary to rate limit the singularity trials instead of sending the raw undamped command directly to Webots

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Before submitting  Confirm that the prediction, results table, three screenshots, joint-rate comparison plot, four answers, and your name are included. Save the personal copy as LastName_Lab04_Report.docx.